***   UTILITIES NOTES   ***

[A] -- WTS30.EXE --

===============

It is a utility for helping user to select Where To Show the

output. User can change the display output to LCD / CRT or LCD & CRT

simultaneously.

[B] For IDE Local BUS ....

-- IDEINS.BAT --

================

IT is a utility for helping user to install the IDE Local BUS

Driver for MicroSoft WINDOWS 3.x .

TO install s1000.386 to a window directory C:\WINDOWS type:

ideins c:\windows

-- ADDDRV.EXE --

================

It will make a copy of WINDOWS's SYSTEM.INI named SYSTEM.100.

In which file, the S1000.386 be installed.

ps.  : It be used in IDEINS.BAT

-- DEVA05S.SYS --

================

DEVA05S.SYS is used to set speed and multi-sector numbers

for both 1st and 2nd IDE channels (for 2nd channel only

speed setting).

It also makes 1st channel multi-sector transfer active to

increase the HDD performance.

To use it, the following command should be append to your

CONFIG.SYS file.

device=[drive]:\[path]\DEVA05S.SYS [1st speed]

[1st multi-sector] [2nd speed]

After reset your system, DEVA05S.SYS will take effect.

If you do not specify speed settings, DEVA05S.SYS will

ask you to fill it.

If you do not specify multi-sector numbers, DEVA05S.SYS

will use default number (20H).

[C] For CD-ROM Driver ....

If you wont to install a CD-ROM, there are some things to do

1. set the CD-ROM I/O port to 230h

2. set LastDrive=Z

3. add the following command to AUTOEXEC.BAT

c:\dos\MSCDEX.EXE /D:MSCD001 /V /L:H

^^^^^^set CD-ROM as H: Drive

Ex.  For a MITSUMI Double Speed CD-ROM, The following two line

have to be added to CONFIG.SYS :

device=c:\utl\MTMCDAS.SYS /D:MSCD001 /V/C /P:230

LastDrive=Z                              ^^^^^^^^

[D] For MOUSE Driver

MOUSE.COM       Mouse driver as a command (call from AUTOEXEC.BAT

or DOS prompt)

MOUSE.SYS       Mouse driver as a device driver ( call from

CONFIG.SYS )

MTUTOR.EXE      Work file for mouse tutorial

MTUTOR.SCR

GTEST.EXE       Work file for mouse test